AQRESS SENSEGRID

# Functional Requirements Document (FRD)

AQRESS SenseGrid — IoT Sensor & Device Management Platform

Functional scope and acceptance requirements for IoT Platform MVP V0.1.

| Document version | 0.1.1 |
| --- | --- |
| Date | 22 August 2026 |
| Primary frontend | React + TypeScript |
| Primary backend | Python + FastAPI + PostgreSQL |

MVP objective: allow a user to register a device, add and configure supported sensors, receive sensor readings, and view live and historical data.

FRD scope This document defines the first usable release: users can sign in, manage sites and devices, add supported sensor types, configure sensor instances, receive readings, and view latest and historical data.

# 1. Purpose

The purpose of the platform is to provide a reusable way to onboard IoT devices and sensors without building a new application for every sensor. The MVP will prove that a user can configure multiple supported sensor types and obtain reliable readings through one common platform.

# 2. Scope

## 2.1 In scope

User authentication

Site management

Device registration and status

Sensor type catalogue with measurement definitions

Add/configure/enable/disable sensor instances

Push configuration to devices and receive acknowledgement

Receive telemetry through MQTT

Persist readings in PostgreSQL

Latest reading and historical reading views

Basic device/sensor error states and audit information

## 2.2 Out of scope for V0.1

Remote switching of industrial equipment

Billing/subscriptions

AI anomaly detection

Predictive maintenance

LoRaWAN network server integration

Full multi-tenant organisation administration

SMS/WhatsApp alerting

Over-the-air firmware updates

# 3. Actors and Roles

| Actor | Description | MVP permissions |
| --- | --- | --- |
| Administrator | Authenticated user with ADMIN role. | Full MVP access, including sensor type catalogue, device credentials and administration. |
| User | Authenticated user with USER role. | Manage authorised sites, devices, sensors and configurations; view readings. |
| Viewer | Authenticated user with VIEWER role. | Read-only dashboard, inventory, device/sensor detail and readings access. |
| IoT Device | Registered device such as an ESP32. | Publish only its telemetry/status/config acknowledgements; subscribe only to its own configuration/command topics. |
| System | FastAPI, PostgreSQL, EMQX and ingestion components. | Validate, persist, authorise, distribute configuration and expose data to the UI. |

# 4. Primary User Journey

User signs in.

User creates or selects a site.

User registers an IoT device.

The device connects and the platform shows it as online.

User opens the device and selects Add Sensor.

User selects a supported sensor type.

The platform displays configuration fields defined for that sensor type.

User saves the sensor and configuration.

The platform sends a versioned configuration to the device.

The device acknowledges successful application.

The device starts publishing readings.

The user sees the latest reading and historical chart.

# 5. Functional Requirements

| ID | Requirement | Statement | Acceptance criterion |
| --- | --- | --- | --- |
| FR-001 | User Authentication | The system shall allow a registered active user to sign in, refresh an authenticated session and sign out. | Access tokens are short-lived; refresh tokens can be revoked server-side and only token hashes are stored. |
| FR-002 | Role Authorisation | The system shall enforce ADMIN, USER and VIEWER permissions together with resource scope checks. | VIEWER cannot mutate data; USER cannot perform administrator-only actions; unauthorised access is rejected. |
| FR-003 | Create Site | The system shall allow a user to create a site with a name and optional location details. | Created site appears in the site list and can host devices. |
| FR-004 | Register Device | The system shall allow a user to register a device with a unique device UID and display name. | A unique device record and credentials are created. |
| FR-005 | Device Connectivity | The system shall determine whether a registered device is online based on MQTT status/heartbeat messages. | Online/offline status and last seen time are visible. |
| FR-006 | Sensor Type Catalogue | The system shall maintain supported sensor types with interface, driver, configuration schema and one or more measurement definitions. | Administrator can create or maintain supported types and their measurement definitions. |
| FR-007 | Add Sensor | The system shall allow a user to add a sensor instance to a selected device. | sensor_uid is unique within the selected device; sensor channels are created from sensor-type measurement definitions. |
| FR-008 | Dynamic Sensor Form | The system shall render configuration fields from the selected sensor type JSON Schema. | A documented JSON Schema subset drives React form generation and authoritative backend validation. |
| FR-009 | Configure Sensor | The system shall allow a user to configure connection parameters and sample interval for a sensor. | Valid configuration is stored as a new version. |
| FR-010 | Configuration Validation | The backend shall validate sensor configuration against the sensor type schema before publication. | Invalid settings are rejected with clear field-level errors. |
| FR-011 | Publish Configuration | The system shall publish an approved sensor configuration to the correct device MQTT topic. | Only the intended device receives its configuration. |
| FR-012 | Configuration Acknowledgement | The system shall receive device acknowledgement for a configuration version. | Sensor/device shows Applied or Failed rather than assuming success. |
| FR-013 | Enable/Disable Sensor | The user shall be able to enable or disable a sensor. | Disabled sensor is reflected in configuration and no longer expected to publish readings. |
| FR-014 | Receive Telemetry | The platform shall accept telemetry from registered devices through MQTT. | Broker authentication and ACL rules restrict each device to its own sensegrid/{device_uid}/ namespace; malformed or unauthorised messages are rejected. |
| FR-015 | Validate Reading | The ingestion service shall validate device ID, sensor ID, measurement code, value type and message structure. | Invalid messages are logged and excluded from normal readings. |
| FR-016 | Store Reading | The system shall store valid readings against the correct sensor channel with device-recorded and server-received timestamps. | recorded_at and received_at are timezone-aware TIMESTAMPTZ values stored in UTC and retrievable by sensor/channel/time range. |
| FR-017 | Raw Payload | The system shall retain the original telemetry payload or equivalent trace data for troubleshooting. | Support staff can inspect the raw message for a reading/event. |
| FR-018 | Latest Reading | The system shall display the latest valid reading for each active sensor measurement. | Latest value includes measurement name/code, unit and timestamp. |
| FR-019 | Historical Readings | The system shall allow users to view readings for one or more measurements of a selected sensor over a date/time range. | Chart and/or table returns the selected period and measurement channels. |
| FR-020 | Live Updates | The platform should update recent sensor values without requiring a full page refresh. | New readings appear through SSE/WebSocket or controlled polling. |
| FR-021 | Device Detail | The system shall show device metadata, online status, last seen, firmware and attached sensors. | Device detail gives one operational view of the device. |
| FR-022 | Sensor Detail | The system shall show sensor status, current configuration, measurement channels, latest values and history. | User can inspect one sensor and all of its measurements end to end. |
| FR-023 | Configuration History | The system shall retain immutable sensor configuration versions after publication. | An edit creates a new incrementing version; only acknowledgement can make that version APPLIED; earlier versions remain auditable. |
| FR-024 | Audit Trail | The system shall record important configuration changes with user and timestamp. | Sensor changes can be traced to who changed what and when. |
| FR-025 | Error Feedback | The UI shall present clear errors for invalid configuration, offline device, failed configuration and unavailable readings. | User can distinguish configuration, connectivity and data issues. |
| FR-026 | Pagination | The system shall paginate inventory lists and bound telemetry queries. | Sites/devices/sensors support page and page_size; readings require a time range/limit and support cursor-based continuation. |
| FR-027 | Time Standard | The system shall use UTC as the persistence standard for all timestamps. | API timestamps use ISO 8601; frontend renders local time without changing stored UTC values. |
| FR-028 | Configuration Schema Standard | The platform shall use a documented subset of JSON Schema for sensor configuration metadata. | The same schema contract can drive React form generation and Python validation. |
| FR-029 | MQTT Topic Authorisation | Each device shall be authorised only for its own MQTT topic namespace. | A device cannot publish or subscribe to sensegrid/{other_device_uid}/ topics. |
| FR-030 | Scoped Uniqueness | The system shall enforce identifiers at the correct ownership scope. | device_uid is globally unique; sensor_uid is unique within a device; measurement keys, sensor channels and configuration versions are unique within their parents. |

# 6. Data Requirements

| Data object | Required minimum | Validation |
| --- | --- | --- |
| User | email, password_hash, role, is_active | email unique; role is ADMIN/USER/VIEWER; timestamps timezone-aware |
| Refresh Token | user_id, token_hash, expires_at, revoked_at | token hash only; user must exist; expiry/revocation enforced |
| Device | device_uid, name, site | device_uid globally unique; name required |
| Sensor Type | code, name, interface_type, driver_key, configuration_schema | code unique; configuration_schema valid against supported JSON Schema subset |
| Measurement Definition | sensor_type_id, key, name, value_type, default_unit | UNIQUE(sensor_type_id, key); sensor type must exist |
| Sensor | sensor_uid, device_id, sensor_type_id, name, enabled | UNIQUE(device_id, sensor_uid); device and type must exist |
| Sensor Channel | sensor_id, measurement_definition_id, name, unit, enabled | UNIQUE(sensor_id, measurement_definition_id); definition must belong to sensor type |
| Configuration | sensor_id, config_version, configuration, status, is_active | UNIQUE(sensor_id, config_version); immutable after publication; validated against type schema |
| Reading | sensor_channel_id, sensor_id, device_id, recorded_at, received_at, typed value | channel/sensor/device match; UTC TIMESTAMPTZ; value type valid |

# 7. Business Rules

| Rule | Definition |
| --- | --- |
| BR-001 | A sensor instance must belong to exactly one registered device. |
| BR-002 | A sensor must reference one supported sensor type. |
| BR-003 | A sensor configuration cannot be published until it passes server-side schema validation. |
| BR-004 | A configuration is not considered Applied until the device acknowledges the same configuration version. |
| BR-005 | A device may publish readings only for sensor IDs assigned to that device. |
| BR-006 | A reading must be stored with both device-recorded time when supplied and server-received time. |
| BR-007 | Disabled sensors remain in history but are not expected to produce current readings. |
| BR-008 | Deleting a sensor must not silently delete historical readings; MVP should prefer deactivation/archiving. |
| BR-009 | Each reading must reference a measurement channel that belongs to the stated sensor. |
| BR-010 | All persisted platform timestamps shall be timezone-aware and stored in UTC using PostgreSQL TIMESTAMPTZ. |
| BR-011 | A published sensor configuration is immutable; a change creates the next configuration version instead of overwriting history. |
| BR-012 | A device may only access MQTT topics under its own sensegrid/{device_uid}/ namespace according to broker ACLs. |
| BR-013 | sensor_uid is unique within a device, not globally; device_uid remains globally unique. |
| BR-014 | The configuration schema shall follow the platform-approved JSON Schema subset and be validated before a sensor type becomes active. |
| BR-015 | Telemetry list responses must be bounded; high-volume reading retrieval shall use time range, limit and cursor continuation. |

# 8. Non-Functional Requirements

| ID | Area | Requirement |
| --- | --- | --- |
| NFR-001 | Security | HTTPS for web/API; MQTT authentication and per-device broker ACLs locally; TLS required for production device transport. Secrets are not committed to source control. |
| NFR-002 | Performance | Typical metadata API responses should target sub-second response under normal MVP load. |
| NFR-003 | Telemetry ingestion | The ingestion design shall support bursty device traffic without blocking user-facing API requests. |
| NFR-004 | Scalability | Readings storage shall support time partitioning and indexed time-range queries. |
| NFR-005 | Availability | A temporary frontend/API interruption shall not require device re-provisioning. |
| NFR-006 | Recoverability | Database backups and restore procedures shall be defined before production use. |
| NFR-007 | Maintainability | Sensor-specific behaviour shall live in sensor types/drivers rather than duplicated application code. |
| NFR-008 | Observability | Application, MQTT and ingestion errors shall be logged with correlation identifiers where practical. |
| NFR-009 | Usability | A normal user shall be able to add a supported sensor through a guided flow without editing code. |
| NFR-010 | Data integrity | Foreign keys, scoped unique constraints, JSON Schema validation and immutable configuration versions shall prevent orphan, duplicated or mismatched data. |
| NFR-011 | Time consistency | All backend persistence and API contracts shall use timezone-aware UTC timestamps; frontend localises for display. |
| NFR-012 | Pagination | Inventory endpoints shall default to bounded page sizes; telemetry endpoints shall never return unbounded historical datasets. |

# 9. MVP Acceptance Scenario

Definition of done An ESP32 is registered, connects to MQTT, receives a user-created DS18B20 configuration, acknowledges it, publishes readings at the configured interval, PostgreSQL stores the readings, and the React UI shows the current value plus history. The same flow must also work for a multi-measurement sensor such as BME280, with temperature, humidity and pressure stored and displayed as separate measurement channels.

# 10. Recommended Prototype Sensors

| Sensor | Interface | Data shape | Why include it |
| --- | --- | --- | --- |
| DS18B20 temperature | OneWire | Single numeric value | Proves simple configurable numeric sensor. |
| BME280 climate | I2C | Temperature + humidity + pressure | Proves one physical sensor can expose multiple measurement channels. |
| Reed/door contact | GPIO | Boolean | Proves non-numeric sensor readings. |

# 11. V0.1.1 Clarifications

These refinements are required before Phase 2 database/authentication implementation; they do not block Phase 1 local infrastructure setup.

Phase 1 remains limited to repository foundation, Docker Compose, PostgreSQL/EMQX services, FastAPI/React skeletons, environment configuration and health checks.

Production deployment remains explicitly out of scope until the local end-to-end MVP has passed integration testing.